AstroStack Manual (v. 0.90  beta)

AstroStack is a freeware program meant for those in the astronomy community working with CCD or video capturing, who want an easy to use program to enhance there results. This program is published for personal use, as is. You are free to distribute this program in the form of the original 'AS_setup.exe' file without alterations of any kind.

Copyright (c) 1999, 2000 Robert Stekelenburg, Netherlands

-------------------------------------------------------------------------------------------------------------------------

1. Introduction

2. First steps

3. What does it all mean?

4. Miscellaneous

1. Introduction

(What is it all about)

I usually take short AVI videos (say upto 15 seconds long, with 30 to 90 pictures) captured from a small B&W CCD video camera connected to my telescope. Almost by hand I used to disassemble the video to get the best pictures in it and combine them with a graphics package to obtain a noise free and sharper image. I did not use more than 4 to 16 pictures at a time. Because an object in a telescope is never truly still, I had to align these pictures by hand. One video often took me a whole evening to process.

AstroStack was originally written by me in 1998 and early 1999 and was completely rewritten late 1999, early 2000. In short AstroStack does all the handwork and more in shorter time with higher quality. I was able to include some features that you won't find in standard graphics packages. Because more and more people are using the (relative to traditional CCD) cheap video imaging possibilities I decided to publish the program in the hope that others find it useful too.

AstroStack is a small, easy to use, yet powerful program to enhance pictures made by CCD and or videocapturing. It is designed to handle full color pictures. It's main purpose is to combine a number of pictures (like a stack of cards, hence the name) made from the same object around the same time. This can be a Windows AVI video or a series of bitmap pictures captured by computer from a video camera attached to your telescope. Combining pictures means that the endresult will have less noise and more detail than any of the original pictures in the sequence can show you. I have often been stunned by the possibilities.

AstroStack has a number of techniques for image enhancement some of which I copied from others and improved upon. This is one of the reasons this program is strictly freeware. Sources are listed below.

AstroStack is so much video oriented that it assumes that output pictures always have a ratio of 4:3. Minimum is 64x48 to 768x576 (full PAL television format). Input pictures or videos do not need to have the same size or ratio. The bigger your result picture the less input pictures you can load and combine. The program has a fixed internal memory use of 32 Mbytes.

Minimum requirements:

Computer: Intel Pentium (or compatible) 133MHz (300 MHz or faster recommended)

RAM: 32 Mbytes (64 Mbytes or more recommended)

Harddisk space: Virtual memory more than 32 Mbytes and some workspace.

Operating system: Windows 95 (but probably works under windows 98, NT and 2000)

AstroStack was written in Delphi 4.0. Some routines where adopted from C++ code and rewritten. I would like the program to remain what it is now: small, easy to use and with a specific purpose. Stuff that you can do well with other programs like printing or final color adjustments will not be included.

2. First steps

(Show me quick how its basics work)

The basic idea is that you load an AVI video or a sequence of bitmap pictures, get a nice result and save this as a bitmap picture. I included some examples to get you started. Once you've followed the instructions you can explore the program on your own with the help of the comprehensive descriptions in the next chapter.

These are the basic steps, follow them to get yourself acquainted with the program. Later you can delve into the features:

- Choose the format of your endresult picture. Default this is 192x144, good for small objects like planets or moon details.

- Now load an AVI video or a sequence of BMP bitmaps. Press the [AVI] or [BMP] button. With a bitmap sequence you have to provide the first and last name of the bitmap from the series e.g. Saturn00.bmp and Saturn25.bmp when your series has 26 bitmaps of Saturn. You can load Saturn3.bmp to Saturn9.bmp, but not Saturn4.bmp to Saturn10.bmp. The numberpart of the name needs to be the same size for all pictures. Press Escape to stop the current loading.

- Press the [Calculate] button

- If you left everything in its default value, an average of all the pictures stacked upon each other will be given in the result window. You probably find it a bit washed out. This is because the details in the original input pictures are not exactly on top of each other. So you have to align all the pictures:

- In the [Align pictures] part press the [Auto] radiobutton, set the [Align with picture] number to 1. Press Go! and sit back and watch! When the process stops press [Calculate] again. This looks much better! Now all the input pictures are aligned with the first input picture. You can also align them with the result picture. Be sure to set the [Align with picture] number to 0 then.

- Now press [Save] and you result is automatically saved under the name Result00.bmp. The number in this name is automatically incremented, so you can use a series of results as input again.

# Examples

Some example pictures of Jupiter have been included to experiment with. I included a short video made with a faulty image intensifier and its darkfield and flatfield images. So you can see and appreciate what they do.

3. What does it all mean?

(Explain all in full detail)

In this chapter I will explain everything. Starting from top to bottom, left to right. Text in brackets [] points to some control in the program like a button, a text field, a radiobutton or an input. In the following text the terms video or bitmap sequence mean the same. Where there are differences in behavior I will highlight them.

[Load AVI video or BMP sequence]

[AVI]

Loads an AVI video into the program.

Any AVI should work, although I tried mainly 8 bit and 24 bit AVI's. Some 8 bit AVI's do not load properly and need to be corrected (see [Correct]).

The maximum amount of pictures depends on the working format you use (shown above in the Result window, default: 192x144 pixels or as adjusted under [Width 4:3]). This maximum nr of pictures you can load is shown in [Input pictures].

Press Escape to stop the current loading.

[BMP]

Loads a sequence of bitmaps

With a bitmap sequence you have to provide the first and last name of the bitmap from the series e.g. Saturn00.bmp and Saturn25.bmp when your series has 26 bitmaps of Saturn. You can load Saturn3.bmp to Saturn9.bmp, but not Saturn4.bmp to Saturn10.bmp. The numberpart of the name needs to be the same size for all pictures. Not all the numbers between the first and last have to be actually present. Press Escape to stop the current loading.

The maximum amount of pictures depends on the working format you use (shown in the Result window, default: 192x144 pixels or as adjusted under [Width 4:3]). This maximum nr of pictures you can load is shown in [Input pictures].

[Correct]

Normally in CCD photography, three correction bitmaps are needed in order to get the best from your CCD image (photometric correction). These are:

- Darkfield frame: An exposure taken under the same conditions as the original video or bitmap sequence, but in total darkness. A darkfield can eliminate the basic noise level (not the noise itself!) or improper settings of the video camera or the capture card of the computer.

- Flatfield frame : An exposure taken under the same conditions and with the same settings as the original images, but against an equally lit background e.g. the blue sky. This will contain the same faults like vignetting and dust spots as the original video.

- Biasframe	   :  An exposure taken with the shortest possible exposure time in total darkness.

For video purposes there seems no need for a biasframe, because of the short exposure times. Apart from this I needed to correct some 8 bit, 384x288 pixel AVI videos, so I misused the Biasframe correction for that. It is not a neat solution but there seemed no space for yet another checkbox.

With the [Correct] button you load or cancel a Darkfield bitmap, a Flatfield bitmap or enable or cancel a correction for some AVI videos. Explanation: Some 384x288 pixel 8 bit videos are shifted 255 pixels to the left when loaded. I have not found the cause of this, nor do I know how other format videos behave in this respect. It looks as if the bitmap color lookuptable is not skipped, when reading the bitmap from the video stream (you can actually see it at the bottom of the loaded pictures). With the [Correct] button load an arbitrary bitmap when asked for and the correction will be applied on loading a video or bitmap sequence.

If one off the above-mentioned corrections are enabled, then the text color of the [Correct] button turns red, otherwise if no correction applies at all the text color is black.

The Darkfield and Flatfield frames need to have the same dimensions as the original video or bitmap sequence that need to be corrected.

Remember to load or enable the corrections BEFORE you load the video/bitmap sequence!

[Full picture]

If you have a video/bitmap sequence with e.g. a planet in a further fully gray or black background, you obviously only want this small object to be picked out of the whole picture. Leave [Full picture] unchecked and this will happen. AstroStack will find the lightest part of the picture, cut this part out and enlarge it with the amount you have chosen.

If you have a picture of the moon or a starfield than set [Full picture] checked. From the middle of the picture a part will be cut and enlarged.  Exactly which part depends on the dimension of the input picture, the result picture and the enlargment you've chosen. See [Miscellaneous] in this chapter for a discussion on this.

[Resample]

Resampling means that a pictures dimension is changed. In this program only enlargement is possible and only in whole units from 1 time (original dimension) to 6 times enlargement. An resample factor of 2 or 3 will be most commonly used. After enlarging and aligning your pictures the details (resolution) in your endresult will be improved.

[Align pictures]

This part of the program allows you to make the often necessary step of aligning pictures as easy as possible. In most cases automatic alignment will suffice. Only in a few cases troublesome pictures need to be hand corrected. First choose [Align with picture] to set the aligning picture (choose 0 to align with the picture in the result window) where all the other pictures are aligned against, then choose the aligning method and then press the [Go!] button. Press the [Stop!] button or the Escape key to abort the process.

[Aligning method]

Choose between manual and automatic alignment:

Manual alignment:

This works as follows:

1. You choose or set the next input picture to align.

2. You move the chosen input picture until its alignment is as good as possible. To check this you have to look at the Statusbar at the bottom of the program. It gives you the nr of the current input picture, the current displacement in comparison with the aligning picture and a value that gives the difference between the two pictures. This value needs to be minimized, it is zero when a picture is aligned with itself i.e. a perfect match.

A few different methods are at your disposal by using either mouse or keys (after you pressed [Go!]).

- Move the mouse into the input picture area, hold down the mouse button and move it around. Let go of the mouse button and double-click to advance to the next picture.

- Use the scrollbars and [Nr of input picture] control

- Use the keyboard. Set numlock on: use PageUp and PageDown and the arrow keys on the keypad to choose an input picture and to move it around. Be sure that no other control (like one of the scrollbars) that can react on these keys,  is active (it is safe to make Combine method [Mean] active for example).

In manual alignment the whole picture is used and this can become quite slow with large formats (384x288 and above).

Automatic alignment:

In this mode the part of the picture that is used in aligning is always 192x144 or smaller. So if the format you are working with is larger e.g. 384x288, then before pressing [Go!], first move the mouse into the input picture area, hold down the mouse button and move it around. You will see an inverted block of 192x144 pixels over the input picture. Move it over the part that is most important to you and release the mouse button. Then press [Go!], sit and watch. Note the numbers of pictures that fail to align. Maybe some manual alignment afterwards can work. Pictures that fail to align within a certain number of steps will be given a quality factor zero [Dont] i.e. they will not be used in the rest of the process.

[Align with picture]

This is the number of the picture the rest will be aligned against. The number zero means that all input pictures will be aligned against the current result picture. Always use the best or sharpest picture.

[Go!]

Alignment will be active, other controls will be disabled to prevent mishaps.

[Stop!]

Alignment will be stopped, other controls will be enabled again.

[Combine pictures]

The series of input pictures you loaded from a video or bitmap sequence need to be combined to an endresult that can be further treated by image enhancement.

[Combine method]

Mean: the mean value of pixel [x,y] of all input pictures scaled by their quality factor is put into pixel [x,y] of the result picture.

Median: the median value of pixel [x,y] of all input pictures is put into pixel [x,y] of the result picture. You can use the number control [mean around median] to gradually go from median value (0) closer to mean value (1 and more).

MinMax: the number control is used to choose when to pick the minimum or the maximum value of all pixels at [x,y].

Multiply: This one is powerful when applied to a small number of pictures (up to the first 7 is the maximum). This gives a very high contrast but the effect is different than applying normal contrast or gamma corrections. The number control is used to control the lightness of the effect. A lower number will give a lighter picture.

[(De)convolution]

With the help of convolution and deconvolution a whole world of possibilities in image enhancement opens up. I use plain bitmaps for the necessary filters and PSF's (Point Spread Functions, see below), so you can produce them with any good painting program. I use the convention of putting an underscore before the name of a PSF bitmap. Bitmaps without underscore are filters used in convoluting.

[Unsharp mask]

First makes an unsharp copy of the image. Then it subtracts this unsharp copy from the original. The result is that only the sharp components are left. These are added to the original to give a sharper endresult. The number controls the strength of this effect. 0 gives the unsharp copy, 2 gives the original, 3 and higher give a stronger amount of the sharp components.

[Deconvolve]

This is stuff you won't find in your everyday paint program. The two routines given here are a member of a family of methods that are used in professional astronomy. The best of these and one of the most complicated seems to be Maximum Entropy Method which was used to improve the Hubble Telescope pictures before it got its eyeglasses fitted. In this program you find two older methods I succeeded to program: the algorithms by van Cittert and Lucy-Richardson.

Van Cittert is good for low contrast objects like planets. Works strongly, but can enhance noise.

Lucy-Richardson’s method is good for high contrast features like the moon and sometimes even seems to decrease noise. This method works stronger if you apply a mild Unsharp mask before using this one.

Both of these methods are iterative i.e. they work for a few times in a row where with each step the result becomes more enhanced. The number of steps (iterations) can be set with the number control.

[Convolve]

When deconvolution is about making pictures better, then convoluting them makes them worse? So why include this one?

Well this method and the filters applied can be used for a multitude of things: making pictures softer, making them sharp, edge detection, gradient detection. Try the examples supplied with this program. For example: a gradient filter finds very subtle differences in brightness in the picture and makes them visible. A weak star or a moon near Saturn that can be missed with one of the other methods can easily be spotted using the proper gradient or edge detection filter.

(The filters used are 8 bit or 24 bit bitmaps, but with a difference: in order to provide negative values that are needed with sharpening and gradient filters the number 128 is subtracted from each pixel value before it is used in the convolute routine. This means that a bitmap pixel that normally holds values from 0=black to 255=white now can provide for values -128 to +127. Load the supplied example filters in your favorite graphics program, enlarge them (for they are very small) and study their colorvalues.

You can switch each of the three methods on and off and thereby combine them. They will work one after another from top to bottom: unsharp mask first, then deconvolution, then convolution.

Remember that the larger the PSF or filter bitmap the slower the operation. The maximum size of the bitmap is 31x31 pixels and the dimensions should be odd numbers.

[Input pictures]

This window shows the current input picture and is also used in aligning operations. It shows how many pictures where loaded and what the maximum number of pictures allowed is.

[Set quality factor]

[Nr. of input picture]

Use this control to set or change the number of the input picture visible. Also used in aligning operations.

[Quality factor]

Set a quality factor for each picture. Its ranges from 'Don't use' to 'Bad' and 'Average' to 'Good'. Pixel values are multiplied with 0, 10, 20 and 40 respectively when combing pictures to an endresult. Setting a quality factor is not automated, except in the automatic alignment procedure.

You can also use keys F1 to F4 to set the quality factor.

[Calculate]

Pressing this button starts the calculations for the result picture in this order:

combine, unsharp mask, then deconvolution, then convolution.

Keep in mind that the larger the dimensions of the result picture or Filter or PSF and the more pictures you have loaded, the longer the calculation time will be. The progressbar and the statusbar keep you informed during the process.

[Save]

Saves the result picture as a 24-bit bitmap with the given dimensions.

[Save]

Just press the button and the current resultpicture as it shows is saved with the name given in the button text. It is standard saved in the result directory unless you change this by using the [Save As] button.

[Save As]

The current resultpicture as it shows, is saved with the name given by you. Uses the result directory by default.

[Result]

The result window shows the endresult after Calculation. The window itself it is 192x144, but smaller or larger images can be stretched or shrinked to its format. In the heading between [] brackets the real dimension of the endresult is given.

[Show result]

[Color Information]

B&W (luminance): shows only the graytones of the endresult.

Color only (hue):  shows the color information in the picture. Where the pictures is white there

is no color, only graytones.

Full color	:  this a combination of hue and luminance and shows a normal color picture.

Saturation level	: shows the level of saturation of the colors. Black means no color i.e.

gray tones, white means fully saturated color. You can change the saturationlevel with the slider.

[Width 4:3]

With this you can change the width (and the height, because their ratio are fixed) of the endresult. It does not make a difference until you load a new AVI or BMP. The ratio width: height is always 4:3, this is standard (PAL) video format.

Note that all the input pictures you load into the program will get the same dimensions. This speeds up internal calculations, but means that the bigger the picture the less input pictures you can load at any time. Also the bigger the picture the slower all the internal operations will be. The maximum size of 768x576 uses 16 times more memory and is 16 times slower than the default 192x144 picture. An operation on the maximum size is slow, even on a 300 MHz PC.

[Full Image]

Check this if you want to see the full image in the 192x144-result window. Smaller images will be stretched and larger images will be shrinked.

[Saturation]

With this slider you can decrease or increase the color saturation level of the image. A value of 100% is the default (no change).

[Histogram functions]

This is a set of display functions to alter or improve the endresult. They are, with one exception (CLAHE) non-destructive, that is you can switch them on and off any time. This means you don't have to recalculate the picture. They work from top to bottom i.e. first Bias and Gain are set, than the LUT is read and at last Map values is performed.

[Bias] and [Gain]

Check this to change brightness (Bias) or contrast (Gain) of the picture.

[Load LUT]

Check this to enable the lookup table (LUT) feature. With the button [Load LUT] you can load a different lookup table. The LUT's are in the PSF directory. They are just files of 256 bytes. The first byte contains the value for black, the last byte contains a new value for white in the original image.

Default the inverse.lut is loaded. Included are gamma corrections, special effects and others.

[CLAHE]

This stands for Contrast Limited Adaptive Histogram Equalization, it is an improved version of AHE (Adaptive Histogram Equalization). In most paint programs you will find a function like Histogram Equalization. This often works on the whole image and I never liked the results it gave. CLAHE divides the picture in 48 (8x6) regions and performs Histogram Equalization on each of them separately. This gives much better and useful results, especially on the moon. The higher the number, the stronger the effect. A number 0 will turn off CLAHE and leaves you with AHE.

CLAHE is destructive i.e. you can't turn it off. To restore the image you have to press [Calculate] again.

[Map values]

This maps the tone scale of 0 (=black) .. 255 (=white) to 16 .. 240. This often works better when printing images on home printers. It also saves ink when your picture has a lot of black background. A likewise operation is used with newspapers photo's.

You could also do this with your painting program (which you need for printing anyway).

[Miscellaneous]

Progress bar

At the far-left bottom you find the progress bar. This informs you about the progress in loading, calculating etcetera and gives a visual aid when manual aligning pictures.

Status bar

At the bottom of the program. This gives all kinds of textual information about the different operations.

Hints

When you move the mouse to a part of the program and hold it still, a short hint text will be shown for a while. In fact this was all the help I intended to give first (how many of you have read this text so far anyway?). So the texts are a bit long and the time they show a bit too short.

AstroStack uses four different directories by default:

Pictures	: contains BMP sequences for input

PSF		: contains the PSF bitmaps and the LUT's

Result		: contains the results after saving, and the darkfield and flatfield images

Videos		: contains the AVI videos for input

You can change them to different directories during the working of the program.

AstroStack doesn't save INSTELLINGEN after closing the program.

Tip: enlarging a set of pictures before aligning them instead of aligning them and enlarging the endresult afterwards, will often result in more detail.

4. Miscellaneous

- Main features and limitations

- Works with windows bitmaps i.e. color and graylevel information is stored as 8 bit. Internally it works with 32 bit bitmaps for Red, Green, Blue and Luminance values. Calculations for (de)convolution are done with 16 bit arithmetic.

- Full color, but picture enhancement like (de)convolution and histogram functions work only on luminance (gray scale) part of the picture. This speeds up things and you hardly notice the difference. On your television set turn down the brightness and turn up the contrast and color to their maximum and you'll see that the sharpness of the picture is in the luminance component, not in the color information.

- Meant for making attractive pictures, not scientific stuff like photometric measurements

- Could be faster

- Version info, comments en bug reports

This is the very first version published, so I call it a 0.9 beta version. It seems to do well, but I haven't tested this on a host of machines. Please do not get overheated when something fails or even crashes. Let me now when and how it occurred and I can try to fix it in the next version. See my email address at the bottom of this document.

Remember that I originally wrote this program, for my own needs. So what I consider user friendly, could be a limitation and annoyance to you.

- Sources and acknowledgement

To the best of my knowledge I have used routines from the public domain. This is why AstroStack must remain freeware.

1. AVI video routines

I use the Video Mci routines to load a video and convert it to internal bitmaps. Available in Delphi.

Copyright (c) 1998, Wolfgang Krug (CapMci20).

2. Resampling/ enlargement of pictures:

From the Graphic Gems series part III, chapter 1.2. Ported from C++ code to Delphi.

Copyright (c) Dale Schumacher

3. Deconvolution

A practical guide to CCD Astronomy by Patrick Martinez and Alain Klotz. A very good book!

4. CLAHE

From the Graphic Gems series part IV, chapter VIII.5. Ported from C++ code to Delphi.

Copyright (c) Karel Zuiderveld

5. Bias and Gain

From the Graphic Gems series part IV, chapter VI.3.

Copyright (c) Christoph Schlick

6. Misc. sources:

- All of the Graphic Gems series for ideas

- Computer Graphics principles and practice 2nd ed. by Foley and van Dam

- Last but not least the Internet.

AstroStack Manual (v. 0.90  beta)

AstroStack is a freeware program meant for those in the astronomy community working with CCD or video capturing, who want an easy to use program to enhance there results. This program is freeware and published for personal use, as is. You are free to distribute this program in the form of the original 'AS_setup.exe' file without alterations of any kind.

=====================================================================

WARNING! THE PROGRAM IS PROVIDED AS IS WITH NO GUARANTEES OF ANY KIND!

USE IT AT YOUR OWN RISK - YOU ARE THE ONLY PERSON RESPONSIBLE FOR

ANY DAMAGE THIS PROGRAM MAY CAUSE - YOU HAVE BEEN WARNED!

=====================================================================

Copyright (c) 1999, 2000 Robert J. Stekelenburg, Netherlands

Send your comments, bug reports and suggestions to: r.stekelenburg@knoware.nl